SparkFun Electronics

Design Guidelines Using Eagle

File stuff:

- Directory names will be: 'Thingy' (no version number or product variation like 'Thingy with chip antenna')

- File names are: Thingy-v10.brd

- Panel names are: Thingy-Panel-v10.brd

- Minor versions will be Thingy-v11, v12, v13. Major will be v20, v30.

- Prototypes start in the sandbox and migrate to the store front specific category once listed on store front.

PCB Layout:

- Create board frame on 0.05” grid. Make the lower left corner start at 0,0.

- Change line width of the board frame to 0.008”.

- Board frame will be square.

- Change top silkscreen color (tPlace) to white. Bottom silk (bPlace) to yellow. And tDocu to mellow yellow.

- Every board should have the full 'sparkfun.com' logo or have the smaller SFE flame. Add the part : LOGO-SFE to your schematic. Put the box in the lower right corner area of the schematic frame.

- Stick parts on 0.05” grid. You MAY NOT break this rule unless you have a very good reason.

- Label any LED with it's purpose (power, status, D4, Lock, etc).

- Label any connector: Vin, Port1, Batt, 5-9V, etc.

- Label pins where applicable: TX, Power, +, Charge, etc.

- Label switches and switch states: On, USB, etc.

- When applicable, when adding labels, move vias and avoid having vias go through the silkscreen. Gold phoenix will bump labels around and make the board look bad if a via goes through the silk.

- Use a tDocu reference line to get pin labels on a straight line.

- Components will be grouped together (the resistors surrounding a transistor in the schematic will also be together on the PCB).

- Using the autorouter is approved for prototypes.

- Hand routing and touchup of the autorouter is expected for production boards.

- 0.020” vias are allowed. 15mil is minimum.

- Use 10mil traces on simple designs. 8mil is acceptable. 7mil is minimum.

- Use thicker traces for power lines where applicable. 10mil=100mA max, 12mil=500mA, and so on.

- Spacing is 8mil between traces and space.

- Use straight lines with 45degree corners. Avoid 90degree corners. T joints are acceptable.

- Use a ground pour on top/bottom layers where applicable.

- Use a 12mil isolation setting on any ground pour. This will help prevent pours from shorting to traces.

- Load the SparkFun.dru file containing

- 12mil Pad restrings

- 10mil Via restrings

- 10mil Copper/Dimension size

- 8mil trace/space

- Via tenting (Mask limit = 25mil)

Schematic Layout:

- All parts will be on 0.1” grid. You MAY NOT alter the grid.

- All GND connections will use GND symbol

- All VCC, 5V, 3.3V, etc will use appropriate power symbol.

- Every schematic will have the part FRAME-LETTER added to it.

- Use two fiducials on each board where applicable. Add the part FIDUCIAL to the schematic. Use the 1x2mm footprint.

- Any board that is larger than ~1” square will probably require standoffs. Add part: STANDOFFs to the schematic. This part has a keepout layer to show you where the head of the standoff screw.

- Add fiducials, standoff components, and logo box to the lower right area of the schematic sheet.

- Use dashed gray lines to separate a complex design into various smaller bits (for example, charge circuit, accelerometer, etc).

Footprints:

- All footprints need >Name (on tNames layer) and >Value (on tValues layer). Size of both will be 0.024”. If you come across a part on a board that doesn't have this, you must change it and save the library.

- All footprints need silkscreen or tDocu indicators showing mechanical sizes, dimensions, or anything weired about the part.

- Silkscreen within a footprint or board should NOT go over pads or metal that will be exposed (it'll flake off easily).

- Every new footprint and part will have a description containing part information and whether the footprint has been proven.

Panelization:

- When panelizing a design, copy original board, paste it into a new PCB. Use a 0.1” grid, and place copies of board exactly 0.020” in between copies (all sides). No indicators for v-scoring are needed.

- Make sure you are aware of any overhanging parts between copies. A bluetooth module antenna could over-run SMD components on the copy next door.

Assembly sheet:

- All assembly sheets will be in PDF form

- All assembly sheets will have version number in tValue text below the design

- Every part will have value next to the component. Text size should be 0.016”

- LEDs will have color indicated

- Any component not to be populated will have a large 'DNP' label on part.

Printing Schematics:

- Print to PDF

- Make sure 'Rotate' is NOT checked

- Select Printer -> PDF995

- Make sure Printer->Preferences are 'Landscape' so that the schematic comes up horizontal when viewing in a PDF viewer.

- Select Sheets : 'All' to print all sheets

- Scale factor 1